# BRIDGE HYDROLOGY REPORT

Hydrological and Hydraulic Analysis for Bridge Design

Bridge Name: Chapani River Bridge

Chainage: 265.312

Location: Lat: 26.983597°, Lon: 85.910333°

Report Date: March 2026

Generated by: Bridge Hydrology Agent v1.0

# 1. CATCHMENT CHARACTERISTICS

| Parameter | Value |
| --- | --- |
| Catchment Area (A) | 31.30 km² |
| Stream Length (L) | 9.44 km |
| Centroidal Length (Lc) | 4.60 km |
| Maximum Elevation (Hmax) | 566.00 m |
| Minimum Elevation (Hmin) | 137.00 m |
| Elevation Difference | 429.00 m |
| Average Slope (S) | 45.4449 |

# 2. RAINFALL FREQUENCY ANALYSIS

## 2.1 Rainfall Statistics

| Number of Years | 36 |
| --- | --- |
| Mean Annual Maximum | 143.24 mm |
| Maximum Recorded | 437.00 mm |
| Minimum Recorded | 23.00 mm |
| Standard Deviation | 83.12 mm |

## 2.2 Goodness-of-Fit Tests

| Distribution | KS Statistic | KS p-value | Chi-Square | Score |
| --- | --- | --- | --- | --- |
| GEV | 0.4501 | 0.0000 | 91.0692 | 0.0000 |
| Gumbel | 0.1201 | 0.6335 | 3.9544 | 0.8920 |
| Normal | 0.1319 | 0.5162 | 6.7426 | 0.6938 |
| Log_Pearson_III | 0.1377 | 0.4618 | 6.0125 | 0.4866 |
| Laplace | 0.1006 | 0.8244 | 0.9161 | 1.1406 |

Best Fitting Distribution: Laplace

## 2.3 Return Period Rainfall Estimates

| Return Period (Years) | Rainfall Depth (mm) |
| --- | --- |
| 2 | 128.30 |
| 5 | 181.94 |
| 10 | 222.52 |
| 20 | 263.10 |
| 50 | 316.74 |
| 100 | 357.32 |
| 200 | 397.89 |

# 3. PEAK DISCHARGE ANALYSIS

| Method | Peak Discharge (m³/s) |
| --- | --- |
| WECS Method | 187.63 |
| Modified Dickens Method | 151.64 |
| B.D. Richards Method | 288.35 |
| Snyder's Method | 326.75 |

## 3.1 Adopted Design Discharge

100-Year Peak Discharge (Q₁₀₀): 326.75 m³/s

Design Discharge (Qdesign): 359.43 m³/s (including 10% climate change factor)

# 4. SCOUR DEPTH CALCULATION

## 4.1 Scour Parameters

| Design Discharge (Q) | 424.78 m³/s |
| --- | --- |
| Bridge Length (L) | 66.40 m |
| Mean Bed Material Size (dmean) | 0.97 mm |
| Silt Factor (Ksf) | 1.70 |
| Blench's Zero Bed Factor (Fb) | 0.60 |

## 4.2 Mean Scour Calculation

| Section | Lacey's (avg q) | Lacey's (max q) | Blench's | Adopted |
| --- | --- | --- | --- | --- |
| Upstream | 0.81 m | 0.92 m | 4.89 m | 4.89 m |
| Existing | 0.80 m | 0.89 m | 4.80 m | 4.80 m |

## 4.3 Pier and Abutment Scour

| Parameter | Upstream | Existing |
| --- | --- | --- |
| Adopted Mean Scour D (m) | 4.89 | 4.80 |
| Scour Depth - Abutment (m) | 6.21 | 6.10 |
| Scour Depth - Pier (m) | 9.78 | 9.60 |
| Scour Level - Abutment (m) | 133.47 | 132.12 |
| Scour Level - Pier (m) | 129.90 | 128.62 |
| Minimum Soffit Level (m) | 141.18 | 139.72 |