# BRIDGE HYDROLOGY REPORT

Hydrological and Hydraulic Analysis for Bridge Design

Bridge Name: Ratu River Bridge

Chainage: 265.312

Location: Lat: 26.983597°, Lon: 85.910333°

Report Date: March 2026

Generated by: Bridge Hydrology Agent v1.0

# 1. CATCHMENT CHARACTERISTICS

| Parameter | Value |
| --- | --- |
| Catchment Area (A) | 88.84 km² |
| Stream Length (L) | 26.93 km |
| Centroidal Length (Lc) | 14.28 km |
| Maximum Elevation (Hmax) | 757.52 m |
| Minimum Elevation (Hmin) | 229.63 m |
| Elevation Difference | 527.89 m |
| Average Slope (S) | 19.6023 |

# 2. RAINFALL FREQUENCY ANALYSIS

## 2.1 Rainfall Statistics

| Number of Years | 36 |
| --- | --- |
| Mean Annual Maximum | 150.11 mm |
| Maximum Recorded | 436.10 mm |
| Minimum Recorded | 69.40 mm |
| Standard Deviation | 79.58 mm |

## 2.2 Goodness-of-Fit Tests

| Distribution | KS Statistic | KS p-value | Chi-Square | Score |
| --- | --- | --- | --- | --- |
| GEV | 0.0954 | 0.8680 | 2.0342 | 1.0489 |
| Gumbel | 0.1535 | 0.3301 | 3.7054 | 0.5393 |
| Normal | 0.2613 | 0.0117 | 18.3141 | 0.0763 |
| Log_Pearson_III | 0.1089 | 0.7457 | 2.8816 | 0.8641 |
| Laplace | 0.1505 | 0.3529 | 1.3044 | 0.6133 |

Best Fitting Distribution: GEV

## 2.3 Return Period Rainfall Estimates

| Return Period (Years) | Rainfall Depth (mm) |
| --- | --- |
| 2 | 127.87 |
| 5 | 180.82 |
| 10 | 227.47 |
| 20 | 283.50 |
| 50 | 377.00 |
| 100 | 466.80 |
| 200 | 577.54 |

## 2.4 IDF (Intensity-Duration-Frequency) Analysis

Figure 2.1: IDF (Intensity-Duration-Frequency) Curves

### 2.4.1 Rainfall Intensity (mm/hr)

| Duration | 2-year | 5-year | 10-year | 50-year | 100-year | 200-year |
| --- | --- | --- | --- | --- | --- | --- |
| 15 min | 52.20 | 73.82 | 92.86 | 153.91 | 190.57 | 235.78 |
| 30 min | 36.91 | 52.20 | 65.66 | 108.83 | 134.75 | 166.72 |
| 1 hr | 26.10 | 36.91 | 46.43 | 76.96 | 95.28 | 117.89 |
| 2 hr | 18.46 | 26.10 | 32.83 | 54.42 | 67.38 | 83.36 |
| 6 hr | 10.66 | 15.07 | 18.96 | 31.42 | 38.90 | 48.13 |
| 12 hr | 7.53 | 10.66 | 13.40 | 22.22 | 27.51 | 34.03 |
| 24 hr | 5.33 | 7.53 | 9.48 | 15.71 | 19.45 | 24.06 |

# 3. PEAK DISCHARGE ANALYSIS

| Method | Peak Discharge (m³/s) |
| --- | --- |
| WECS Method | 397.63 |
| Modified Dickens Method | 386.19 |
| B.D. Richards Method | 593.28 |
| Snyder's Method | 629.76 |

## 3.1 Adopted Design Discharge

100-Year Peak Discharge (Q₁₀₀): 629.76 m³/s

Design Discharge (Qdesign): 692.74 m³/s (including 10% climate change factor)

# 4. SCOUR DEPTH CALCULATION

## 4.1 Scour Parameters

| Design Discharge (Q) | 818.69 m³/s |
| --- | --- |
| Bridge Length (L) | 226.17 m |
| Mean Bed Material Size (dmean) | 2.80 mm |
| Silt Factor (Ksf) | 2.90 |
| Blench's Zero Bed Factor (Fb) | 0.80 |
| Average Discharge Intensity (q_avg) | 4.245 m²/s |
| Maximum Discharge Intensity (q_max) | 5.943 m²/s |
| Water Surface Elevation (HFL/WSE) | 219.06 m |

## 4.2 Mean Scour Calculation

| Method | Scour Depth (m) |
| --- | --- |
| Lacey's (avg q) | 0.54 m |
| Lacey's (max q) | 0.60 m |
| Blench's | 2.80 m |
| Adopted | 2.80 m |

## 4.3 Pier and Abutment Scour

| Parameter | Value |
| --- | --- |
| Adopted Mean Scour D (m) | 2.80 |
| Scour Depth - Abutment (m) | 3.56 |
| Scour Depth - Pier (m) | 5.60 |
| Scour Level - Abutment (m) | 215.50 |
| Scour Level - Pier (m) | 213.46 |
| Minimum Soffit Level (m) | 220.56 |
| Water Surface Elevation (HFL/WSE) | 219.06 |